CONFIGS-GENERAL INFORMATION - The alternate configurations (CONFIGS) section describes the establishment and manipulation of alternate versions or configurations of the lens database. These alternate configurations are stored with the lens database. Some of the commands described here are CMD level commands issued at the CMD level. These CMD level commands are described here instead of in the CMD section because they act to modify, or in other ways manipulate, these alternate configurations. The other commands described here are CONFIGS input level and UPDATE CONFIGS level commands which can only be issued at one or both of these levels. The alternate configuration database provides a way for surface and non-surface dependent lens parameter values as well as special surface parameter values to be set to values different from those values established in the main lens database and the main special surface database . The lens database is described in the LENS manual section. Special surface database is described in the SPECIAL SURFACES manual section.

THE CONFIGS DATABASE - The alternate configuration (CONFIGS) database consists of a sequence of UPDATE LENS, UPDATE SPSRF and CMD level commands which are  stored in memory and on disk with the "current lens" data. There can be up to 75 configurations defined in a lens database including the regular lens database which is always the main configuration or configuration #1. This means there can be 74 alternate configurations. In each alternate configuration, there can be up to 2000 individual command entries.

CURRENT AND PERMANENT LENS - There is always a "current lens" which was described in the LENS section. There is also always a lens called the "permanent lens". As long as there are no alternate configurations defined, the "permanent lens" is identical to the "current lens". When alternate configurations are defined, the "current lens" is only identical to the "permanent lens" when the main configuration, designated as CFG 1, is the active configuration. If a configuration other than this main configuration becomes the active configuration, then the UPDATE LENS,  UPDATE SPSRF and CMD level commands, which are stored in the alternate configuration database, are applied to the "permanent lens" in order to generate a new "current lens". This new "current lens" is no longer identical to the "permanent lens" but is the result of the commands stored in the alternate configuration database acting or operating upon the "permanent lens". The order in which the alternate configuration commands are applied to the "permanent" lens in order to create the new "current" lens is:

1.	Update Lens level commands are applied,

2.	then all Update Special Surface level commands are applied,

3.	and finally, all CMD level commands are applied.

CREATING A NEW CONFIGS DATABASE

CONFIGS - The "CONFIGS" command causes the program to leave the CMD level and enter the CONFIGS input level. The alternate configuration database is wiped clean and is ready for new input. Between "CONFIGS" and "EOS" or "END", any CONFIGS input level command may be entered. The "CONFIGS" command also wipes clean any optimization and tolerance definitions which were in effect prior its issuance.

EOS or END - The "EOS" or "END" command, issued from the CONFIGS level, causes the program to return to the CMD level. The alternate configuration database is left in memory and is ready for analysis.

MODIFYING A CONFIGS DATABASE

UPDATE CONFIGS or U CF - The "UPDATE CONFIGS" command, or its abbreviated form "U CF", causes the program to leave the CMD level and enter the UPDATE CONFIGS level. The alternate configuration database is opened and is ready for modification. Between "UPDATE CONFIGS" or "U CF" and "EOS" or "END", any UPDATE CONFIGS level command may be entered.

EOS or END - The "EOS" or "END" command, issued from the UPDATE CONFIGS level, causes the program to return to the CMD level. The alternate configuration database is left in memory and is ready for analysis.

CONFIGS AND UPDATE CONFIGS SPECIFIC COMMANDS

CFG , i - The "CFG" command, issued from the CONFIGS or UPDATE CONFIGS level, designates that configuration "i" will be the configuration to which all the following CONFIGS or UPDATE CONFIGS commands are to be applied. Configuration "i" remains the configuration for input until another CFG command is entered or until the CONFIGS or UPDATE CONFIGS levels are exited via the "EOS" or "END" command. Valid values for "i" range from "2" to "75". If this command is not issued, all alternate configuration input will be made to configuration number 2.

CFG - The "CFG" command, issued with no numeric input, simply causes the current configuration number to be displayed.

HOW COMMANDS WITHIN CONFIGS WORK - After the CONFIGS or the UPDATE CONFIGS level has been entered and the configuration number for input has been designated, either by way of the "CFG , i" command or by default, then specifying changes to the main lens database and/or the special surface database is trivial. All that need be done is to enter selected CMD, UPDATE LENS or UPDATE SPSRF level commands in exactly the same way as would have been done from outside the CONFIGS level. These commands will be stored and later executed when configurations are switched at the CMD level.

EXAMPLES

EXAMPLE 1 (A CMD LEVEL CHANGE) - The current lens is set up in mode FOCAL. It is desired to define configuration 3 as mode AFOCAL. The following commands are used:

UPDATE CONFIGS (or CONFIGS)

CFG , 3

MODE AFOCAL

EOS or END (This "EOS" or "END" refers to UPDATE CONFIGS)

The current lens database now has an alternate configuration number 3 attached to it. Using the CMD level command "CFG , 3" switches the lens into configuration 3. The CMD level command "MODE" will display the message "AFOCAL". Switching back to the main configuration with the CMD level command "CFG , 1" returns the lens to configuration number 1. The CMD level command "MODE" will now display the message "FOCAL" as expected.

EXAMPLE 2 (AN UPDATE LENS LEVEL CHANGE) - The current lens has been set up and surface number 5 has a radius of 234.567 lens units. It is desired to have the radius of this surface be 500.00 lens units in configuration 2. The command sequence will be:

UPDATE CONFIGS (or CONFIGS)

CFG , 2 (this is optional since CFG 2 is the default)

UPDATE LENS (or U L)

CHG,5

RD , 500

EOS or END (This "EOS" or "END" refers to "U L" )

EOS or END (This "EOS" or "END" refers to "UPDATE CONFIGS")

The current lens database now has an alternate configuration number 2 attached to it. Using the CMD level command "CFG , 2" switches the lens into configuration 2. The CMD level command "RTG , 5" will display the radius, thickness and glass at surface 5 with radius = 500. Switching back to the main configuration with the CMD level command "CFG , 1" returns the lens to configuration number 1. The CMD level command "RTG , 5" will now display the radius, thickness and glass at surface 5 with radius = 234.567.

EXAMPLE 3 (AN UPDATE SPSRF LEVEL CHANGE) - The current lens has been set up and surface number 6 has special surface definition assigned to it. Coefficient C2 = 2.34D-5. It is desired to have the C2 coefficient of this surface be = 0 in configuration 10. The command sequence will be:

UPDATE CONFIGS (or CONFIGS)

CFG , 10

UPDATE SPSRF (or U SP)

C2 , 6 , 0

EOS or END (This "EOS" or "END" refers to "U SP")

EOS or END (This "EOS" or "END" refers to "UPDATE CONFIGS")

The current lens database now has an alternate configuration number 10 attached to it. Using the CMD level command "CFG , 10" switches the lens into configuration 10. The CMD level command "PRSPR , 6" will display coefficient values at surface 6. C2 will be = 0. Switching back to the main configuration with the CMD level command "CFG , 1" returns the lens to configuration number 1. The CMD level command "PRSPR , 6" will now display the coefficient values at surface 6. C2 will be back to 2.34D-5.

EXAMPLE 4 (EXAMPLES 1, 2 AND 3 ALL TOGETHER) - All of the changes could have been done at once with the following command sequence:

UPDATE CONFIGS (or CONFIGS)

CFG , 3

MODE AFOCAL

CFG , 2

UPDATE LENS

CHG,5

RD , 500

EOS or END (This "EOS" or "END" refers to "U L")

CFG , 10

UPDATE SPSRF

C2 , 6 , 0

EOS or END (This "EOS" or "END" refers to "U SP"

EOS or END (This "EOS" or "END" refers to "UPDATE CONFIGS)

ALLOWED COMMANDS WITHIN CONFIGS - All UPDATE LENS commands (except "INS" and "DEL")  and all of the UPDATE SPSRF are valid for use in defining alternate configurations. Consult the LENS and SPECIAL SURFACES manual sections for command syntax. The following CMD level commands are also valid within the CONFIGS or UPDATE CONFIGS level:

MODE ( FOCAL, UFOCAL, AFOCAL or UAFOCAL),

THERM (SHAPE, THICK, SPACE, GLASS, GAS, or specific gas names) , i , j , T , coeff,

PRES (GAS, or specific gas names) , i , j , P , coeff,

SPTWT , 1, 2 , 3 , 4 , 5  and SPTWT2 , 6, 7 , 8 , 9 , 10,

and the first order f-number, exit pupil radius and magnification hold commands:

FNBX, FNBX HLD, FNBY, FNBY HLD, ERX, ERX HLD, ERY, ERY HLD, MAGX, MAGX,

HLD, MAGY, MAGY HLD, FLDS MAX, FLDS FRA, FLDS POS, FLDS ANG, FLDS and

SPSRF (ON or OFF) .,i

CMD LEVEL CONFIGS MANIPULATION

CONFIGS DISPLAY COMMANDS

CF , i - The "CF" command, when issued at the CMD level, causes the "i"th configuration data to be displayed. If "i" is not explicitly entered, all of the alternate configuration data from all currently defined configurations will be displayed. Each item in the alternate configuration database is numbered sequentially within each configuration. These sequential position values are the first items listed in this data display. Should an item need to be removed from the alternate configuration database, the item is referred to by the configuration number and entry number.

REMOVE , i , j , k - The "REMOVE" command can be used to manually delete alternate configuration data items number "j" through the "k" in configuration number "I".

DELCFG , i - The "DELCFG" command deletes the "i"th configuration from the current configuration database attached to the lens.

CFG , i - The "CFG" command, when issued at the CMD level, causes the "i"th configuration to become the "current" configuration represented as a new "current" lens. Only the "current" configuration in the form of the "current" lens can be used in ray trace or other forms of analysis.

DEZOOM , i - The "DEZOOM" command, when issued at the CMD level, causes the "i"th configuration to become the "permanent lens" and the "current" lens. All other alternate configuration data is deleted! The "DEZOOM" command also wipes clean any optimization and tolerance definitions which were in effect prior its issuance.

AUTOMATIC DATA MANAGEMENT - Many items which can be made part of an alternate configuration definition are mutually exclusive. A "TH" definition is mutually exclusive to a "PY" or a "PIKUP TH" definition. Mutually exclusive items will be automatically deleted with the last item entered taking precedence. The exceptions to this rule are the "THERM" and "PRES" commands. Since the user may wish to enter several identical "THERM" and "PRES" commands in order to produce a specific lens configuration, no automatic removal of "THERM" or "PRES" commands will be performed. Only through the use of the "REMOVE" command can individual "THERM" or "PRES" commands be removed from alternate configuration data.

HEXAGON/ACCOS-V INPUT - Between the issuance of a CONFIGS or UPDATE CONFIGS command and the EOS or END command, the following HEXAGON/ACCOS-V commands may be entered. They are automatically converted into regular program commands.

(parameter name) , j , v - The "(parameter name)" command (from the following table), when issued at the configs level, causes the parameter on the "j"th surface to take on the value "v".

| parameter name | parameter |
| --- | --- |
| CV | profile curvature |
| RD | profile radius |
| CVR | toric curvature |
| RDR | toric radius |
| CC | conic constant |
| XD | x-decenter |
| YD | y-decenter |
| ALPHA | alpha tilt |
| BETA | beta tilt |
| GAMMA | gamma tilt |
| AD | 4th order aspheric coefficient |
| AE | 6th order aspheric coefficient |
| AF | 8th order aspheric coefficient |
| AG | 10th order aspheric coefficient |
| RN | refractive index (this results in a null input) |
| DF | dispersion factor (this results in a null input) |
| TH | axial thickness |
| TEMP | medium temperature (this results in a null input) |
| GLALPHA | medium thermal expansion coefficient (this results in a null input) |